1. Introduction to CCNA

What is CCNA?

CCNA = Cisco Certified Network Associate

CCNA is a networking certification from Cisco that teaches the fundamentals of:

- Networking

- IP addressing

- Ethernet

- Switching

- Routing

- VLANs

- Wireless networking

- Network security

- Network automation

What will you learn in CCNA?

Networking Fundamentals

↓

IP Addressing

↓

Switching

↓

VLANs

↓

Routing

↓

Network Services

↓

Security

↓

Automation

Why learn CCNA?

CCNA knowledge is useful for roles such as:

- Network Support Engineer

- NOC Engineer

- Network Engineer

- Infrastructure Engineer

- Cloud Network Engineer

- Network Security Engineer

2. What is a Network?

A network is a collection of two or more devices connected together so that they can communicate and share resources.

Simple example

PC1 ─────── Switch ─────── PC2

│

│

Printer

PC1 can communicate with PC2 and access the printer through the network.

Why do we use networks?

Networks allow us to:

- Share files

- Share printers

- Access the Internet

- Communicate between computers

- Share applications

- Access servers

- Transfer data

Real-world example

When you send a WhatsApp message:

Your Phone

↓

Wi-Fi / Mobile Network

↓

Internet

↓

WhatsApp Infrastructure

↓

Internet

↓

Receiver's Phone

A network allows the devices and systems involved to communicate.

3. What is an IP Address?

IP = Internet Protocol

An IP address is a logical address assigned to a device on an IP network.

It helps identify a device and determine where IP packets should be delivered.

Example IPv4 address

192.168.1.10

An IPv4 address contains 32 bits and is normally written as four decimal numbers called octets.

192     .     168     .     1     .     10

↓            ↓             ↓          ↓

Octet        Octet         Octet      Octet

Each octet ranges from:

0 – 255

Example network

PC1                  PC2

192.168.1.10         192.168.1.20

│                    │

└────── Switch ──────┘

Both devices can communicate because they are configured within the same IP network, assuming appropriate subnetting and configuration.

IPv4 vs IPv6

IPv4

192.168.1.10

- 32-bit address

- Four octets

- Decimal notation

IPv6

2001:db8:abcd::10

- 128-bit address

- Hexadecimal notation

- Designed to provide a much larger address space

4. Types of Addressing

In basic CCNA networking, you should understand unicast, broadcast, multicast, and anycast.

4.1 Unicast

One sender → One receiver

PC1 ───────────→ PC2

Example:

PC1 sends data specifically to PC2.

Example

192.168.1.10 → 192.168.1.20

This is one-to-one communication.

4.2 Broadcast

One sender → All devices in the broadcast domain

┌── PC2

│

PC1 ── Switch├── PC3

│

└── PC4

PC1 sends a broadcast, and eligible devices in the same broadcast domain receive it.

IPv4 broadcast example

192.168.1.255

For a /24 network:

Network:   192.168.1.0

Broadcast: 192.168.1.255

4.3 Multicast

One sender → Multiple selected receivers

┌── PC2

│

Server ──────┼── PC3

│

└── PC4

Only devices that have joined the multicast group receive the traffic.

Commonly used for things such as:

- Streaming

- Routing protocols

- Video distribution

4.4 Anycast

One sender → One suitable/nearest member of a group

Anycast is commonly associated with IPv6 and services distributed across multiple locations.

Conceptually:

┌── Server A

Client ──────┤

├── Server B

│

└── Server C

The network directs the client toward an appropriate destination.

5. Transmission Modes

Transmission mode describes the direction in which data can flow between communicating devices.

There are three common modes:

- Simplex

- Half-duplex

- Full-duplex

5.1 Simplex

One-way communication only

Sender ─────────→ Receiver

The receiver cannot send data back through the same communication channel.

Example

Traditional television broadcasting:

TV Station ─────────→ TV

5.2 Half-Duplex

Both devices can communicate, but not at the same time.

Device A ─────→ Device B

←──────

One device transmits while the other waits.

Example

Walkie-talkie:

Person A: "Over"

Person B: "Over"

Only one person normally speaks at a time.

5.3 Full-Duplex

Both devices can transmit and receive simultaneously.

Device A ←────────→ Device B

Example

Modern switched Ethernet:

PC ←────────────→ Switch

Data can flow in both directions simultaneously.

Important CCNA point

Modern switched Ethernet networks normally operate in full-duplex mode, which eliminates Ethernet collisions on those links.

6. Network Topology

A network topology describes how devices and connections are arranged in a network.

There are two major perspectives:

Topology

│

├── Physical Topology

│

└── Logical Topology

7. Physical Topology

Physical topology describes the actual physical arrangement of devices, cables, ports, and connections.

Example:

PC1 ─────┐

PC2 ─────┤

PC3 ─────┤ Switch

PC4 ─────┘

It answers:

"How are the devices physically connected?"

Things considered include:

- Network cables

- Switches

- Routers

- Access points

- Device locations

- Physical interfaces

8. Logical Topology

Logical topology describes how data flows through the network, regardless of the physical arrangement.

Example:

PC1

↓

VLAN 10

↓

Switch

↓

VLAN 10

↓

PC2

It answers:

"How does data logically move through the network?"

Logical topology can involve:

- VLANs

- IP addressing

- Routing

- Switching

- Data flow

Simple difference

| Physical Topology | Logical Topology |
| --- | --- |
| Physical connections | Data flow |
| Cables | VLANs |
| Ports | IP networks |
| Device locations | Routing |
| Hardware arrangement | Logical communication |

9. Types of Network Topology

The major topology types are:

- Bus

- Star

- Ring

- Mesh

- Tree

- Hybrid

9.1 Bus Topology

All devices share a common backbone.

PC1 ─── PC2 ─── PC3 ─── PC4

Common Backbone

Advantages

- Simple

- Less cable required

Disadvantages

- Backbone failure can affect the network

- Difficult troubleshooting

- Mostly obsolete in modern LANs

9.2 Star Topology

All devices connect to a central device, usually a switch.

PC1

│

│

PC2 ───── Switch ───── PC3

│

│

PC4

Advantages

- Easy to manage

- Easy troubleshooting

- Failure of one cable normally affects only one device

- Common in modern Ethernet LANs

Disadvantage

If the central switch fails, connected devices can lose network connectivity.

9.3 Ring Topology

Each device connects to two neighboring devices, forming a ring.

PC1 ─── PC2

│        │

│        │

PC4 ─── PC3

Advantages

- Predictable data path

Disadvantages

- A failure can affect communication depending on the implementation

- Less common for modern enterprise LANs

9.4 Mesh Topology

Devices have multiple connections to other devices.

Full Mesh

Every device connects to every other device.

A

/|\

/ | \

B--|--C

\ | /

\|/

D

Advantages

- High redundancy

- High availability

- Multiple paths

Disadvantages

- Expensive

- Complex

- Requires many connections

Used where redundancy is important, such as certain WAN and data-center designs.

9.5 Tree Topology

A hierarchical structure of interconnected devices.

Core

/    \

Switch    Switch

/  \       /  \

PC1  PC2   PC3  PC4

Common in enterprise networks.

9.6 Hybrid Topology

Combination of two or more topology types.

For example:

Star + Mesh

Large enterprise networks commonly use combinations of different topologies.

10. Types of Networks

Networks can be classified according to their geographical or functional scope.

10.1 PAN

PAN = Personal Area Network

Small network around an individual.

Example:

Phone ←── Bluetooth ──→ Earbuds

Range is generally very small.

10.2 LAN

LAN = Local Area Network

Network covering a small geographical area such as:

- Home

- Office

- School

- Computer lab

Example:

PC ── Switch ── PC

│

PC

10.3 WLAN

WLAN = Wireless Local Area Network

A LAN that uses wireless communication, usually Wi-Fi.

Laptop

)))

)))

Access Point

)))

)))

Phone

10.4 MAN

MAN = Metropolitan Area Network

Covers a larger area such as a city or metropolitan region.

Building A ─────┐

│

Building B ─── MAN

│

Building C ─────┘

10.5 WAN

WAN = Wide Area Network

Connects networks across large geographical areas.

Example:

Office A ───── Internet/WAN ───── Office B

The Internet is the world's largest interconnected network.

10.6 SAN

SAN = Storage Area Network

A specialized high-speed network used to connect servers to storage systems.

Common in:

- Data centers

- Enterprise environments

- Storage infrastructure

Quick Comparison

| Network | Full Form | Typical Scope |
| --- | --- | --- |
| PAN | Personal Area Network | Person |
| LAN | Local Area Network | Building |
| WLAN | Wireless LAN | Wireless local area |
| MAN | Metropolitan Area Network | City |
| WAN | Wide Area Network | Country/Global |
| SAN | Storage Area Network | Storage infrastructure |

11. Types of Network Hardware

Network hardware refers to physical devices used to connect, forward, transmit, secure, or extend network communication.

Important devices for CCNA include:

NIC

↓

Hub

↓

Bridge

↓

Switch

↓

Router

↓

Access Point

↓

Repeater

↓

Firewall

12. NIC

NIC = Network Interface Card

A NIC allows a device to connect to a network.

Examples:

- Ethernet NIC

- Wireless NIC

PC ─── NIC ─── Network

Important

A NIC has a MAC address used for Layer 2 communication.

13. MAC Address

MAC = Media Access Control

A MAC address is a hardware/link-layer address associated with a network interface.

Example:

00:1A:2B:3C:4D:5E

It is typically 48 bits for Ethernet.

IP vs MAC

| IP Address | MAC Address |
| --- | --- |
| Logical address | Link-layer/hardware address |
| Layer 3 | Layer 2 |
| Used for routing | Used for local Ethernet delivery |
| Can change | Usually associated with the NIC |

14. Hub

A hub is a basic Layer 1 networking device.

When it receives a signal on one port, it repeats it out the other ports.

PC1

│

PC2 ─── Hub ─── PC3

│

PC4

Problems

- One collision domain

- Shared bandwidth

- No MAC address intelligence

- Generally operates in half-duplex

Modern status

❌ Hubs are largely obsolete.

15. Bridge

A bridge connects network segments and makes forwarding decisions based on MAC addresses.

LAN 1 ─── Bridge ─── LAN 2

A bridge is a Layer 2 device.

Important

Modern switches essentially provide the functionality of a multiport bridge.

16. Switch

A switch is primarily a Layer 2 device that forwards Ethernet frames based on MAC addresses.

PC1 ──┐

PC2 ──┤

PC3 ──┤ Switch

PC4 ──┘

Switch learns MAC addresses

Example:

MAC Address       Port

-----------       ----

AAAA.AAAA.AAAA    Fa0/1

BBBB.BBBB.BBBB    Fa0/2

CCCC.CCCC.CCCC    Fa0/3

Switch advantages

- Efficient forwarding

- Each access port is normally its own collision domain

- Supports VLANs

- Usually operates in full-duplex on modern Ethernet links

17. Router

A router connects different IP networks.

LAN 1

192.168.1.0/24

│

Router

│

LAN 2

192.168.2.0/24

A router primarily operates at Layer 3.

Router responsibilities

- Connect different networks

- Make routing decisions

- Forward IP packets

- Support routing protocols

- Provide services such as NAT depending on platform/configuration

Example

192.168.1.0/24

│

Router

│

192.168.2.0/24

18. Access Point

AP = Access Point

An access point provides wireless network connectivity to wireless clients.

Laptop

)))

)))

PC ── Switch ── AP

)))

)))

Phone

It connects wireless devices to a wired network.

19. Repeater

A repeater receives a signal and regenerates/repeats it to extend the communication distance.

Device ─── Signal ─── Repeater ─── Signal ─── Device

A repeater primarily operates at Layer 1.

Example

Used when a signal needs to travel farther than the normal physical medium allows.

20. Firewall

A firewall controls network traffic according to security rules.

Internet

│

Firewall

│

Internal Network

It can control traffic based on factors such as:

- Source IP

- Destination IP

- Ports

- Protocols

- Connection state

- Application information, depending on the firewall

Example:

Internet ──X──→ Internal Server

if the firewall policy blocks that traffic.

🧠 Important Hardware Comparison

| Device | Main Layer | Main Function |
| --- | --- | --- |
| NIC | L1/L2 | Network connectivity |
| Repeater | L1 | Regenerate/repeat signals |
| Hub | L1 | Repeat traffic to ports |
| Bridge | L2 | Forward frames using MAC addresses |
| Switch | L2 | Forward frames using MAC addresses |
| Router | L3 | Connect different IP networks |
| Access Point | L2 primarily | Wireless connectivity |
| Firewall | L3–L7 depending on type | Traffic security/control |

🔥 Important Concepts to Remember

MAC vs IP

MAC Address

↓

Layer 2

↓

Local network delivery

IP Address

↓

Layer 3

↓

Communication between networks

Switch vs Router

Switch

↓

Connects devices within/among VLANs at Layer 2

↓

Uses MAC addresses

Router

↓

Connects different IP networks

↓

Uses IP addresses

Access Port vs Trunk Port

Access Port

↓

Normally carries one VLAN

Trunk Port

↓

Carries traffic for multiple VLANs

🎯 Complete CCNA Foundation Map

Students should understand this sequence before moving deeper into Cisco configuration:

CCNA

│

┌──────────┴──────────┐

│                     │

Networking              Devices

│                     │

┌─────┼─────┐        ┌─────┼─────┐

│     │     │        │     │     │

IP   MAC  Transmission NIC Switch Router

Address      Modes

│

├── IPv4

└── IPv6

Topology

│

├── Physical

└── Logical

│

├── Star

├── Bus

├── Ring

├── Mesh

├── Tree

└── Hybrid

Network Types

│

├── PAN

├── LAN

├── WLAN

├── MAN

├── WAN

└── SAN