1. Transmission Modes

Transmission mode describes the direction in which data can travel between two devices.

There are three types:

1. Simplex

Data travels in only one direction.

Sender ───────────────→ Receiver

Example: Traditional TV broadcasting.

- Sender → Receiver

- Receiver cannot send data back

2. Half-Duplex

Data can travel in both directions, but not at the same time.

Device A ─────────→ Device B

Device A ←───────── Device B

Example: Walkie-talkie.

One person speaks while the other waits.

3. Full-Duplex

Data can travel in both directions simultaneously.

Device A ←────────────→ Device B

Example: Modern switched Ethernet.

A computer can send and receive data at the same time.

Quick comparison

| Mode | Direction | Example |
| --- | --- | --- |
| Simplex | One way | TV broadcast |
| Half-duplex | Both, one at a time | Walkie-talkie |
| Full-duplex | Both simultaneously | Modern Ethernet |

2. Wired Transmission

Wired transmission uses a physical cable to carry data.

PC ───────── Cable ───────── Switch

Advantages

- Reliable

- High speed

- Lower interference than wireless in many environments

- Better physical control

- Suitable for enterprise networks

Common wired media

- Twisted-pair cable

- Coaxial cable

- Fiber-optic cable

3. Types of Network Cables

3.1 Twisted-Pair Cable

The most common Ethernet cable used in LANs.

It contains pairs of copper wires twisted together.

Copper wires

↓

Twisted pairs

↓

Ethernet cable

Common connector:

RJ-45 / 8P8C modular Ethernet connector

Used to connect:

- PC → Switch

- Router → Switch

- Access Point → Switch

3.2 Coaxial Cable

Contains:

- Central conductor

- Insulation

- Metallic shielding

- Outer jacket

┌─────────────────────┐

│ Outer Jacket        │

│  ┌───────────────┐  │

│  │ Shield        │  │

│  │  ┌─────────┐  │  │

│  │  │ Conductor│  │  │

│  │  └─────────┘  │  │

│  └───────────────┘  │

└─────────────────────┘

Commonly associated with:

- Cable TV

- Broadband cable Internet

- Older networking technologies

4. Fiber-Optic Cable

Fiber uses light to transmit data rather than electrical signals.

Device ───── Fiber ───── Device

↓

Light

Advantages

- Very high bandwidth

- Long-distance transmission

- Resistant to electromagnetic interference

- Suitable for backbone and data-center connections

Two major types

Single-mode fiber — SMF

- Long distances

- Very small core

- Common in telecom and long-haul networks

Multimode fiber — MMF

- Shorter distances

- Larger core

- Common in buildings and data centers

5. Ethernet Cable Categories — Cat Types

Cat = Category

Cat cables are categories of twisted-pair Ethernet cabling.

| Cable | Typical capability | Common use |
| --- | --- | --- |
| Cat5 | Up to 100 Mbps | Legacy |
| Cat5e | Up to 1 Gbps | Common LAN |
| Cat6 | Up to 1 Gbps / higher over shorter distances | Modern LAN |
| Cat6a | Up to 10 Gbps | Enterprise |
| Cat7 | Higher-spec cabling | Specialized deployments |
| Cat8 | Up to 25/40 Gbps over short distances | Data centers |

Important

For basic CCNA:

Cat5e and Cat6 are the most important to recognize.

Maximum common Ethernet distance

For standard twisted-pair Ethernet:

Maximum channel length ≈ 100 meters

Typically:

90 m permanent link

+

10 m patch cables

=

100 m channel

6. Wired Cable Types: Straight-Through and Crossover

Students may encounter these terms in networking labs.

Straight-through

Historically used to connect different types of devices.

Example:

PC ─────── Switch

Crossover

Historically used to connect similar device types.

Example:

PC ─────── PC

or:

Switch ─────── Switch

Important modern point

Many modern Ethernet interfaces support Auto-MDI/MDIX, so the device can automatically detect and accommodate the cable wiring.

7. Wireless Transmission

Wireless transmission sends data through electromagnetic/radio waves instead of a physical cable.

Laptop

)))

))) Radio

)))

Access Point

│

│ Ethernet

▼

Switch

Common wireless technologies include:

- Wi-Fi

- Bluetooth

- Cellular networks

- Microwave

- Satellite

For CCNA, Wi-Fi is especially important.

8. 2.4 GHz Wi-Fi

2.4 GHz is one of the common Wi-Fi frequency bands.

Advantages

- Longer range than 5 GHz in typical indoor environments

- Better penetration through walls

- Widely supported

Disadvantages

- More interference

- Lower available channel capacity

- Often more congested

Sources of interference can include:

- Bluetooth devices

- Microwave ovens

- Other Wi-Fi networks

- Wireless devices

Important channels

In many regulatory domains, the commonly recommended non-overlapping 2.4 GHz Wi-Fi channels are:

Channel 1

Channel 6

Channel 11

This is especially relevant in traditional 20 MHz channel planning.

9. 5 GHz Wi-Fi

The 5 GHz band provides more channel options and is commonly used for higher-performance Wi-Fi.

Advantages

- More available channels

- Generally less congestion than 2.4 GHz

- Higher performance in many environments

Disadvantages

- Shorter effective range in many indoor environments

- More attenuation through walls

Simple comparison

| Feature | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Range | Generally higher | Generally lower |
| Wall penetration | Better | Lower |
| Interference | Higher | Generally lower |
| Available channels | Fewer | More |
| Typical performance | Lower | Higher |

Note: Modern Wi-Fi also uses 6 GHz with Wi-Fi 6E/Wi-Fi 7 equipment, but 2.4 and 5 GHz are still important CCNA fundamentals.

10. Wireless Security — WPA, WPA2 and WPA3

These are generations of Wi-Fi security standards.

The evolution is:

WEP

↓

WPA

↓

WPA2

↓

WPA3

WEP

WEP = Wired Equivalent Privacy

Old Wi-Fi security standard.

- Uses RC4

- Has serious security weaknesses

- Easily compromised

- Obsolete

❌ Never use WEP in a modern network.

11. WPA

WPA = Wi-Fi Protected Access

WPA was introduced as an improvement over WEP.

Key technology

TKIP = Temporal Key Integrity Protocol

WPA was mainly a transitional improvement before WPA2.

❌ WPA/TKIP should not be used for modern deployments.

12. WPA2

WPA2 = Wi-Fi Protected Access 2

A major improvement over WPA.

WPA2-Personal

Uses:

PSK = Pre-Shared Key

Example:

Wi-Fi Password

↓

WPA2-Personal

WPA2 commonly uses:

AES-CCMP

WPA2-Enterprise

Uses enterprise authentication, commonly involving:

802.1X + RADIUS

Client

↓

Access Point

↓

RADIUS Server

↓

Authentication

13. WPA3

WPA3 = Wi-Fi Protected Access 3

Newer Wi-Fi security standard.

For WPA3-Personal, authentication uses:

SAE = Simultaneous Authentication of Equals

SAE provides stronger protection against offline password-guessing attacks than the WPA2-Personal PSK handshake.

Best practice

WPA3

↓

Preferred

WPA2-AES

↓

Fallback when WPA3 isn't supported

WEP / WPA-TKIP

↓

Avoid

14. OSI Model

OSI = Open Systems Interconnection

The OSI model is a 7-layer conceptual model used to understand how network communication works.

Layer 7 → Application

Layer 6 → Presentation

Layer 5 → Session

Layer 4 → Transport

Layer 3 → Network

Layer 2 → Data Link

Layer 1 → Physical

Easy mnemonic

From Layer 7 → Layer 1:

All People Seem To Need Data Processing

15. OSI Layer 7 — Application

Provides network services to applications.

Examples:

- HTTP/HTTPS

- DNS

- SMTP

- FTP

- SSH

User Application

↓

Application Layer

Example:

When you open a website, HTTP/HTTPS is involved at the application layer.

16. OSI Layer 6 — Presentation

Responsible conceptually for:

- Data formatting

- Translation

- Encryption/decryption

- Compression

Example:

Encryption

Data formatting

Compression

In modern TCP/IP implementations, these functions are often handled by application libraries/protocols rather than a distinct layer.

17. OSI Layer 5 — Session

Manages communication sessions between applications.

Responsibilities conceptually include:

- Establishing sessions

- Maintaining sessions

- Terminating sessions

Again, in modern TCP/IP networking, session functionality is often implemented within applications and transport protocols rather than as a separate layer.

18. OSI Layer 4 — Transport

Provides end-to-end transport services.

Main protocols:

TCP

TCP = Transmission Control Protocol

Provides:

- Reliable delivery

- Sequencing

- Acknowledgments

- Flow control

- Connection-oriented communication

UDP

UDP = User Datagram Protocol

Provides:

- Connectionless communication

- Lower overhead

- No built-in delivery guarantee

PDU

At the Transport Layer:

TCP → Segment

UDP → Datagram

19. OSI Layer 3 — Network

Responsible for logical addressing and routing.

Main protocol:

IP = Internet Protocol

Devices:

- Routers

- Layer 3 switches

Examples:

IPv4

IPv6

PDU

Packet

IP Header + Data

20. OSI Layer 2 — Data Link

Responsible for local network delivery.

Functions include:

- Ethernet framing

- MAC addressing

- Frame delivery

- Error detection with mechanisms such as FCS

Devices:

- Switch

- Bridge

- Network interface

PDU

Frame

Ethernet Header + Data + FCS

21. OSI Layer 1 — Physical

Responsible for transmitting raw bits over the physical medium.

Examples:

- Copper cable

- Fiber-optic cable

- Radio signals

- Connectors

- Electrical/light/radio signaling

PDU

Bits

101101001011...

22. OSI Model Summary

| Layer | Name | Main Function | Examples |
| --- | --- | --- | --- |
| 7 | Application | Network services to applications | HTTP, DNS, SMTP |
| 6 | Presentation | Format, encryption, compression | TLS-related functions, encoding |
| 5 | Session | Manage communication sessions | Session management |
| 4 | Transport | End-to-end transport | TCP, UDP |
| 3 | Network | IP addressing & routing | IPv4, IPv6 |
| 2 | Data Link | Frames & MAC addressing | Ethernet, 802.11 |
| 1 | Physical | Bits & signals | Copper, fiber, radio |

23. TCP/IP Model

The TCP/IP model is the practical networking model underlying the Internet.

The commonly taught 4-layer TCP/IP model is:

4 → Application

3 → Transport

2 → Internet

1 → Network Access

24. TCP/IP Application Layer

Combines the functions represented by OSI:

OSI Layer 7 → Application

OSI Layer 6 → Presentation

OSI Layer 5 → Session

Examples:

- HTTP/HTTPS

- DNS

- DHCP

- SSH

- SMTP

- FTP

25. TCP/IP Transport Layer

Corresponds approximately to OSI Layer 4.

Main protocols:

- TCP

- UDP

Responsibilities:

- End-to-end communication

- Segmentation

- Reliability with TCP

- Flow control

- Port numbers

26. TCP/IP Internet Layer

Corresponds approximately to OSI Layer 3.

Main protocol:

- IP

Responsibilities:

- Logical addressing

- Packet delivery

- Routing

Examples:

- IPv4

- IPv6

- ICMP

27. TCP/IP Network Access Layer

Corresponds approximately to:

OSI Layer 2 + Layer 1

Responsible for:

- Ethernet

- Wi-Fi

- MAC addressing

- Frames

- Physical transmission

- Media access

28. OSI vs TCP/IP

| OSI | TCP/IP |
| --- | --- |
| 7 layers | 4 layers |
| Application | Application |
| Presentation | Application |
| Session | Application |
| Transport | Transport |
| Network | Internet |
| Data Link | Network Access |
| Physical | Network Access |

Easy mapping

OSI                    TCP/IP

7 Application ───┐

6 Presentation   ├──→ Application

5 Session ───────┘

4 Transport ─────────→ Transport

3 Network ───────────→ Internet

2 Data Link ──────┐

1 Physical ───────┴──→ Network Access

29. Structure of Data — Encapsulation

This is one of the most important concepts in networking.

When an application sends data, each layer adds its own information.

This process is called encapsulation.

Application

↓

Data

↓

Transport

↓

Segment

↓

Network

↓

Packet

↓

Data Link

↓

Frame

↓

Physical

↓

Bits

30. Bits

At the Physical Layer, data is transmitted as bits.

A bit has one of two binary values:

0 or 1

Example:

101100101010

The actual physical representation depends on the medium.

For example:

- Copper → electrical signals

- Fiber → light

- Wireless → radio signals

31. Frame

At the Data Link Layer, data is called a frame.

For Ethernet, a simplified structure is:

┌──────────┬──────────┬──────┬──────────┬─────┐

│ Dest MAC │ Src MAC  │Type  │   Data   │ FCS │

└──────────┴──────────┴──────┴──────────┴─────┘

Important fields

Destination MAC

Identifies the intended Layer 2 destination.

Source MAC

Identifies the Layer 2 sender.

Type/Length

Indicates the encapsulated protocol or length, depending on Ethernet framing.

Data

Contains the payload.

FCS

FCS = Frame Check Sequence

Used for error detection.

32. Packet

At the Network Layer, data is called a packet.

For IPv4:

┌─────────────────────────────┐

│ IPv4 Header                 │

│ Source IP                   │

│ Destination IP              │

│ Other fields                │

├─────────────────────────────┤

│ Transport Segment/Datagram  │

└─────────────────────────────┘

Important addresses:

Source IP

Destination IP

Example:

192.168.1.10

↓

192.168.2.20

Routers primarily use the destination IP address to make forwarding decisions.

33. Segment

At the Transport Layer, when TCP is used, the data unit is called a segment.

Simplified:

┌─────────────────────┐

│ TCP Header          │

│ Source Port         │

│ Destination Port    │

│ Sequence Number     │

│ ACK Number          │

├─────────────────────┤

│ Application Data    │

└─────────────────────┘

Important fields include:

- Source port

- Destination port

- Sequence number

- Acknowledgment number

- Flags

- Window information

UDP

For UDP, the correct term is generally:

UDP Datagram

not TCP segment.

34. Complete Encapsulation Example

Suppose a user opens an HTTPS website.

Application

│

│ HTTPS Data

▼

Transport

│

│ TCP Segment

▼

Network

│

│ IP Packet

▼

Data Link

│

│ Ethernet Frame

▼

Physical

│

│ Bits

▼

Network Medium

So remember:

Application Data

↓

Segment

↓

Packet

↓

Frame

↓

Bits

35. Decapsulation

At the receiving device, the reverse process occurs.

Bits

↓

Frame

↓

Packet

↓

Segment

↓

Application Data

This is called decapsulation.

Complete picture

SENDER                                  RECEIVER

Application Data

↓

Segment

↓

Packet

↓

Frame

↓

Bits

│

│ Network

▼

Bits

↓

Frame

↓

Packet

↓

Segment

↓

Application Data

⭐ Important Exam Points

Remember these five PDUs:

Layer 4 → Segment     (TCP)

Layer 3 → Packet

Layer 2 → Frame

Layer 1 → Bits

For UDP:

Layer 4 → Datagram

Remember the devices:

Layer 1 → Hub / Repeater

Layer 2 → Switch / Bridge

Layer 3 → Router

Remember the addresses:

Layer 2 → MAC Address

Layer 3 → IP Address

Layer 4 → Port Number

The big picture

NETWORK COMMUNICATION

Application → Data

↓

Transport → Segment / Datagram

↓

Network → Packet

↓

Data Link → Frame

↓

Physical → Bits

↓

Network

↓

Physical → Bits

↓

Data Link → Frame

↓

Network → Packet

↓

Transport → Segment / Datagram

↓

Application → Data

This encapsulation → transmission → decapsulation concept is the foundation for understanding what happens when a packet travels through a PC → switch → router → Internet → destination path.