1. What is an IP Address?

IP = Internet Protocol

An IP address is a logical address assigned to a network interface so that devices can communicate using IP.

Think of an IP address like a postal address for a device on a network.

PC1

IP Address: 192.168.1.10

│

▼

Switch

│

▼

Router

An IP address helps identify:

- Which network the device belongs to

- Which host/device it represents within that network

2. IPv4

IPv4 = Internet Protocol version 4

IPv4 uses a 32-bit address.

It is normally written as four decimal numbers separated by dots.

Example

192.168.1.10

Each section is called an octet.

192 . 168 . 1 . 10

│     │    │    │

Octet Octet Octet Octet

Each octet contains:

8 bits

Therefore:

4 × 8 = 32 bits

Binary representation

192.168.1.10

11000000.10101000.00000001.00001010

Each octet can have a value from:

0 – 255

3. Network Portion and Host Portion

An IPv4 address consists conceptually of:

Network Portion + Host Portion

The subnet mask tells us where the network portion ends and the host portion begins.

Example:

IP Address:    192.168.1.10

Subnet Mask:   255.255.255.0

CIDR notation:

192.168.1.10/24

Here /24 means the first 24 bits are the network portion.

Network              Host

<----------------> <-------->

192.168.1           .10

4. Private IP Address

A private IP address is used inside private networks such as:

- Home networks

- Office networks

- Schools

- Data centers

- Cloud virtual networks

Private IPv4 addresses are not directly routable across the public Internet.

Private IPv4 ranges

| Class | Private Range |
| --- | --- |
| A | 10.0.0.0/8 |
| B | 172.16.0.0/12 |
| C | 192.168.0.0/16 |

Examples

10.0.0.10

172.16.5.20

192.168.1.10

These are private addresses.

5. Public IP Address

A public IP address is an address that can be routed on the public Internet.

Example:

203.0.113.10

Public IP addresses are generally allocated through Internet addressing authorities and service providers.

Example network

Internet

│

Public IP

203.x.x.x

│

Router

┌─────┴─────┐

│           │

PC1           PC2

192.168.1.10   192.168.1.20

Private        Private

Your internal devices can use private IP addresses while the router/firewall uses a public IP for Internet connectivity.

6. Private IP vs Public IP

| Feature | Private IP | Public IP |
| --- | --- | --- |
| Used | Internal networks | Internet |
| Internet routable | ❌ No | ✅ Yes |
| Example | 192.168.1.10 | ISP-assigned public address |
| Common use | PCs, servers, printers | Internet-facing services |
| Address uniqueness | Can be reused in different private networks | Must be globally unique at a given time |

7. IPv4 Address Classes

Historically, IPv4 addresses were divided into Class A, B, C, D, and E.

This is called classful addressing.

⚠️ Modern networks primarily use CIDR (Classless Inter-Domain Routing) rather than relying on address classes. However, students should still understand the historical classes because they are commonly taught in networking fundamentals.

Class A

First octet:

1 – 126

Default subnet mask:

255.0.0.0

CIDR:

/8

Example:

10.0.0.1

Designed for very large networks.

Class B

First octet:

128 – 191

Default subnet mask:

255.255.0.0

CIDR:

/16

Example:

172.16.1.10

Historically intended for medium-sized networks.

Class C

First octet:

192 – 223

Default subnet mask:

255.255.255.0

CIDR:

/24

Example:

192.168.1.10

Historically intended for smaller networks.

Class D

Range:

224 – 239

Used for:

Multicast

Example:

224.0.0.1

Class D does not represent normal unicast host addressing.

Class E

Range:

240 – 255

Historically reserved for experimental/future use.

8. IPv4 Classes — Quick Table

| Class | First Octet | Default Mask | Purpose |
| --- | --- | --- | --- |
| A | 1–126 | /8 | Large networks |
| B | 128–191 | /16 | Medium networks |
| C | 192–223 | /24 | Smaller networks |
| D | 224–239 | N/A | Multicast |
| E | 240–255 | N/A | Experimental |

Important exceptions

127.0.0.0/8

is reserved for loopback.

For example:

127.0.0.1

is commonly used to refer to the local host.

9. IPv6

IPv6 = Internet Protocol version 6

IPv6 was developed primarily to solve the limitation of IPv4 address space.

IPv4:

32 bits

IPv6:

128 bits

Example IPv6 address

2001:db8:1234:5678::10

IPv6 uses hexadecimal numbers separated by colons.

10. Why IPv6?

IPv4 provides approximately:

2³²

possible addresses.

IPv6 provides:

2¹²⁸

possible addresses.

That is an enormously larger address space.

IPv6 also provides

- Larger address space

- Address autoconfiguration capabilities

- No broadcast

- Multicast and anycast support

- Improved support for modern IP networking

11. IPv4 vs IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32-bit | 128-bit |
| Format | Decimal | Hexadecimal |
| Example | 192.168.1.10 | 2001:db8::10 |
| Broadcast | Yes | No |
| Address space | Smaller | Extremely large |
| Address configuration | DHCP/static/manual | SLAAC, DHCPv6, static |

12. NAT

NAT = Network Address Translation

NAT translates IP addresses, commonly between private IPv4 addresses and public IPv4 addresses.

Why do we use NAT?

Primarily:

- To allow private IPv4 hosts to access the Internet

- To conserve public IPv4 addresses

- To hide internal addressing from direct Internet exposure

13. Basic NAT Example

Imagine:

PC

192.168.1.10

│

▼

Router

│

Public IP

203.0.113.10

│

▼

Internet

PC has a private IP:

192.168.1.10

The Internet cannot directly route that private address.

The router translates it to a public address.

192.168.1.10

↓

NAT

↓

203.0.113.10

↓

Internet

14. Common Types of NAT

For CCNA fundamentals, know these:

Static NAT

One private IP ↔ One public IP.

192.168.1.10

↕

203.0.113.10

Common for specific servers that need a consistent public address.

Dynamic NAT

Private addresses are translated using a pool of public addresses.

Private IPs

↓

NAT Pool

↓

Public IPs

PAT

PAT = Port Address Translation

Also commonly called NAT overload.

Multiple private devices share a single public IPv4 address by using different source port numbers.

PC1 ── 192.168.1.10 ─┐

│

PC2 ── 192.168.1.20 ─┤

├── NAT ── 203.0.113.10

PC3 ── 192.168.1.30 ─┘

This is extremely common for home and enterprise Internet access.

15. ARP

ARP = Address Resolution Protocol

ARP is used in IPv4 networks to discover the MAC address associated with a known IPv4 address on the local network.

Example

PC1 knows:

Destination IP:

192.168.1.20

But Ethernet communication needs a destination MAC address.

PC1 sends an ARP request:

"Who has 192.168.1.20?"

PC2 replies:

"192.168.1.20 is at

AA:BB:CC:DD:EE:FF"

PC1 can then send the Ethernet frame to that MAC address.

16. ARP Process

PC1

IP: 192.168.1.10

MAC: AA-AA-AA

│

│ ARP Request

│ "Who has 192.168.1.20?"

▼

Switch

│

▼

PC2

IP: 192.168.1.20

MAC: BB-BB-BB

│

│ ARP Reply

│ "I have it!"

▼

PC1

PC1 stores the result in its ARP cache.

17. ARP Cache

On a Windows computer:

arp -a

This displays the local ARP cache.

Example:

Internet Address      Physical Address

192.168.1.1           aa-bb-cc-dd-ee-ff

192.168.1.20          11-22-33-44-55-66

Important

ARP is used for:

IPv4 Address → MAC Address

IPv6 does not use ARP. IPv6 uses Neighbor Discovery Protocol (NDP) instead.

18. Remote Access

Remote access allows an administrator to connect to and manage a network device from another location.

Instead of physically connecting to the router or switch:

Administrator

│

│ Network

▼

Router / Switch

The administrator can remotely:

- Configure devices

- Troubleshoot problems

- Check interfaces

- Monitor network status

- Change configurations

Common remote-access protocols include:

- Telnet

- SSH

19. Telnet

Telnet = Teletype Network

Telnet allows remote access to a device using a command-line interface.

Typical architecture:

Administrator PC

│

│ Telnet

▼

Cisco Switch

Telnet problem

Telnet sends information, including credentials, without encryption.

Conceptually:

Username ───────────────→

Password ───────────────→

Commands ───────────────→

An attacker who can capture the traffic may be able to read sensitive information.

Therefore:

❌ Telnet should not be used for secure production administration.

It may still be encountered in old networks and basic educational labs.

20. SSH

SSH = Secure Shell

SSH provides secure remote command-line access.

Administrator PC

│

│ SSH

▼

Cisco Router/Switch

SSH encrypts the communication between the administrator and the network device.

SSH protects

- Username/password exchange

- Commands

- Configuration information

- Management traffic

Therefore:

✅ SSH is preferred over Telnet for remote device administration.

21. Telnet vs SSH

| Feature | Telnet | SSH |
| --- | --- | --- |
| Full form | Teletype Network | Secure Shell |
| Encryption | ❌ No | ✅ Yes |
| Security | Weak | Stronger |
| Default TCP port | 23 | 22 |
| Production use | ❌ Avoid | ✅ Recommended |
| Remote CLI | ✅ | ✅ |

22. CCNA Remote Access Example

A Cisco administrator might connect to a switch like this:

Telnet

telnet 192.168.1.10

SSH

ssh admin@192.168.1.10

The exact syntax and authentication configuration depend on the device and software.

23. Complete Concept Map

IP ADDRESSING

│

┌──────────┴──────────┐

│                     │

IPv4                  IPv6

│                     │

32-bit address         128-bit address

│

┌──────┴──────┐

│             │

Private        Public

│             │

└──────┬──────┘

│

NAT

│

Private ↔ Public

│

ARP

│

IPv4 → MAC

│

Remote Access

┌───┴───┐

Telnet    SSH

❌         ✅

⭐ Important CCNA Points to Remember

IP Address

Logical address used for communication at Layer 3.

IPv4

32-bit addressing system written in dotted-decimal notation.

Private IP

Used inside private networks and not directly routable on the public Internet.

Public IP

Globally routable IPv4 address used for Internet communication.

NAT

Translates IPv4 addresses, commonly private addresses to public addresses.

ARP

Resolves an IPv4 address to a MAC address on the local network.

Telnet

Remote CLI access, but unencrypted.

SSH

Secure, encrypted remote CLI access.

Most important flow

Application

↓

IP Address

↓

ARP finds MAC (IPv4 local delivery)

↓

Ethernet Frame

↓

Switch

↓

Router

↓

NAT (when applicable)

↓

Internet

For your students, the next natural practical sequence after these notes is IPv4 addressing → subnet mask → subnetting → default gateway → ARP → Cisco switch management → Telnet → SSH → VLANs.