1. Virtual Local Area Network (VLAN)

VLAN = Virtual Local Area Network

A VLAN is a logical division of a Layer 2 switched network.

Instead of putting every device into one broadcast domain, we can divide the network into multiple VLANs.

Without VLANs

Switch

┌──────┼──────┐

PC1    PC2    PC3

PC4    PC5    PC6

One Broadcast Domain

All devices are in the same VLAN/broadcast domain by default.

With VLANs

Switch

┌────────┴────────┐

│                 │

VLAN 10           VLAN 20

STUDENTS           TRAINERS

│   │              │   │

PC1 PC2            PC3 PC4

Now the switch has two logical networks:

VLAN 10 → Students

VLAN 20 → Trainers

Devices in different VLANs are isolated at Layer 2 unless a Layer 3 device provides routing between them.

2. Why Do We Use VLANs?

VLANs provide:

1. Segmentation

Separate different groups of devices.

Example:

VLAN 10 → Students

VLAN 20 → Trainers

VLAN 30 → Management

2. Security

Devices in one VLAN are logically separated from devices in another VLAN.

3. Smaller Broadcast Domains

Broadcast traffic is contained within the VLAN.

4. Better Network Management

Devices can be grouped logically rather than only according to their physical location.

3. VLAN Example

Suppose a college has:

Students:

PC1, PC2, PC3

Trainers:

PC4, PC5

Management:

PC6

We can create:

VLAN 10 → STUDENTS

VLAN 20 → TRAINERS

VLAN 30 → MANAGEMENT

Even if the PCs are connected to the same physical switch, they can belong to different logical networks.

4. Creating a VLAN on Cisco Switch

Example:

enable

configure terminal

vlan 10

name STUDENTS

exit

vlan 20

name TRAINERS

exit

Verify:

show vlan brief

Example output:

VLAN   Name

----   ----------------

10     STUDENTS

20     TRAINERS

5. Access Port

An access port is a switch port normally assigned to one VLAN.

Example:

PC1

│

│

Fa0/1

│

Switch

Configure:

interface fa0/1

switchport mode access

switchport access vlan 10

Now:

Fa0/1 → VLAN 10

Typical use

Access ports are commonly used for:

- PCs

- Printers

- IP phones, with additional configuration

- Servers

- End-user devices

6. Access Port Example

Switch

┌───────┼───────┐

│       │       │

Fa0/1   Fa0/2   Fa0/3

│       │       │

PC1     PC2     PC3

VLAN10  VLAN10  VLAN20

Configuration:

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 10

interface fa0/3

switchport mode access

switchport access vlan 20

7. Trunking

Trunking allows a single physical link to carry traffic belonging to multiple VLANs.

Example:

VLAN 10

│

│

┌────┴────┐

│   SW1   │

└────┬────┘

│

│ Trunk

│

┌────┴────┐

│   SW2   │

└────┬────┘

│

│

VLAN 10

The trunk can carry:

VLAN 10

VLAN 20

VLAN 30

...

over the same physical link.

8. Why Do We Need Trunking?

Imagine two switches:

SW1                    SW2

│                      │

PC1                    PC3

VLAN 10                VLAN 10

We want PC1 and PC3 to communicate as members of VLAN 10.

The switches need a link that can carry VLAN 10 between them.

Now imagine:

VLAN 10

VLAN 20

VLAN 30

Instead of creating one physical cable for every VLAN, we can use one trunk link.

SW1 ================= SW2

Trunk

VLAN 10,20,30

This is one of the main benefits of VLAN trunking.

9. Trunk Port

A trunk port is a switch port configured to carry traffic for multiple VLANs.

Example:

SW1 Fa0/24

│

│ Trunk

│

SW2 Fa0/24

Cisco configuration:

interface fa0/24

switchport mode trunk

Verify:

show interfaces trunk

10. Access Port vs Trunk Port

| Feature | Access Port | Trunk Port |
| --- | --- | --- |
| VLANs | Normally one | Multiple |
| Typical connection | PC | Switch-to-switch |
| Purpose | End device connectivity | VLAN transport |
| VLAN tagging | Usually frames are untagged toward end device | Uses VLAN tagging |
| Example | PC → Switch | Switch → Switch |

Easy way to remember

ACCESS

↓

One VLAN

TRUNK

↓

Multiple VLANs

11. VLAN Tagging

How does a trunk know which VLAN a frame belongs to?

Modern Ethernet trunking commonly uses:

IEEE 802.1Q

Also called:

Dot1Q

A VLAN tag is inserted into the Ethernet frame to identify the VLAN.

Conceptually:

Normal Ethernet Frame

┌────────┬────────┬──────┐

│ Dest   │ Source │ Data │

└────────┴────────┴──────┘

802.1Q Tagged Frame

┌────────┬────────┬──────┬──────┐

│ Dest   │ Source │ VLAN │ Data │

└────────┴────────┴──────┴──────┘

The VLAN ID tells the receiving switch which VLAN the frame belongs to.

12. Native VLAN

On an 802.1Q trunk, the native VLAN is the VLAN whose frames are sent untagged by default.

Cisco switches commonly use:

VLAN 1

as the default native VLAN.

However, for security and good operational practice, many organizations configure an unused VLAN as the native VLAN.

Example:

interface fa0/24

switchport trunk native vlan 999

Both ends of the trunk should have matching native VLAN configuration.

13. Allowed VLANs

A trunk does not necessarily need to carry every VLAN.

We can specify which VLANs are allowed.

Example:

interface fa0/24

switchport trunk allowed vlan 10,20,30

Now the trunk carries:

VLAN 10

VLAN 20

VLAN 30

but not other VLANs.

Best practice

Only allow VLANs that are actually required on the trunk.

This reduces unnecessary Layer 2 traffic and can improve security.

14. Dynamic Trunking

Cisco switches can use DTP to dynamically negotiate trunking.

DTP = Dynamic Trunking Protocol

DTP is a Cisco proprietary protocol used to negotiate whether a link should operate as a trunk.

Common switchport modes include:

access

trunk

dynamic desirable

dynamic auto

15. Dynamic Auto

switchport mode dynamic auto

The interface will not actively try to form a trunk, but it can become a trunk if the neighboring side requests/negotiates trunking.

Think:

"I'm willing to become a trunk."

16. Dynamic Desirable

switchport mode dynamic desirable

The interface actively attempts to negotiate a trunk.

Think:

"I want to form a trunk."

17. Dynamic Trunking Example

Suppose:

SW1                    SW2

Fa0/24                 Fa0/24

Desirable              Auto

The two sides can negotiate a trunk.

SW1 ================= SW2

TRUNK

18. Dynamic Trunking — Important Table

| SW1 | SW2 | Result |
| --- | --- | --- |
| Access | Access | Access |
| Trunk | Trunk | Trunk |
| Trunk | Auto | Trunk |
| Trunk | Desirable | Trunk |
| Desirable | Auto | Trunk |
| Desirable | Desirable | Trunk |
| Auto | Auto | Access/non-trunk |
| Auto | Desirable | Trunk |

Exact behavior can depend on the switch platform and DTP support/configuration.

Production best practice

Don't rely on dynamic negotiation for production switch-to-switch links.

Prefer explicitly configuring:

switchport mode trunk

And, where appropriate, disable DTP negotiation:

switchport nonegotiate

19. Port Security

Port Security is a Layer 2 switch security feature used to control which MAC addresses are allowed on a switch port.

It helps prevent unauthorized devices from using an access port.

Example:

PC ───── Fa0/1

│

Switch

We can configure the port so that only specific MAC addresses are allowed.

20. Why Use Port Security?

Imagine an office:

Employee PC

│

Fa0/1

│

Switch

Someone disconnects the employee PC and connects their own laptop.

Without port security:

Unknown Laptop ── Switch

✅

With port security configured:

Unknown Laptop ── Switch

❌

The switch can detect an unauthorized source MAC address and take an action.

21. Basic Port Security Configuration

First configure the port as an access port:

interface fa0/1

switchport mode access

Enable port security:

switchport port-security

Specify an allowed MAC address:

switchport port-security mac-address 0011.2233.4455

Now only that MAC address is allowed on the port.

22. Sticky MAC Address

Instead of manually typing the MAC address, Cisco can dynamically learn a MAC address and add it to the port-security configuration.

Command:

switchport port-security mac-address sticky

Example:

interface fa0/1

switchport mode access

switchport port-security

switchport port-security mac-address sticky

The switch learns the source MAC address.

Conceptually:

PC

MAC = AAAA.BBBB.CCCC

↓

Switch

↓

Sticky MAC learned

You can verify it with:

show port-security interface fa0/1

23. Maximum Number of MAC Addresses

We can specify how many MAC addresses are allowed.

Example:

switchport port-security maximum 2

This means:

Maximum allowed MAC addresses = 2

Useful when a port legitimately needs multiple devices, such as certain IP phone + PC scenarios, although those setups should be designed carefully.

24. Port Security Violation

A violation occurs when a port receives traffic from a MAC address that is not allowed under its port-security configuration.

Cisco provides violation modes.

The three commonly taught modes are:

Shutdown

Restrict

Protect

25. Shutdown Mode

switchport port-security violation shutdown

This is the common/default violation mode on many Cisco IOS platforms.

When a violation occurs, the port is placed into an error-disabled state.

Unauthorized MAC

↓

Violation

↓

Port

↓

ERR-DISABLED

An administrator must investigate and recover the port.

26. Restrict Mode

switchport port-security violation restrict

The switch drops traffic from unauthorized MAC addresses and generates violation information such as counters/logging, depending on platform.

The port remains operational.

27. Protect Mode

switchport port-security violation protect

Unauthorized frames are dropped.

The port remains operational, but violation logging/counters are more limited than with restrict mode.

28. Port Security Violation Comparison

| Mode | Unauthorized Traffic | Port Status | Violation Information |
| --- | --- | --- | --- |
| Protect | Dropped | Remains up | Limited |
| Restrict | Dropped | Remains up | More visibility/counters |
| Shutdown | Dropped | Error-disabled | Strong response |

Easy memory

PROTECT

↓

Drop

RESTRICT

↓

Drop + report/count

SHUTDOWN

↓

Drop + disable port

29. Complete Port Security Example

Suppose:

PC1 → Fa0/1

We want:

- Access port

- Maximum 1 MAC address

- Sticky MAC learning

- Shutdown on violation

Configuration:

enable

configure terminal

interface fa0/1

switchport mode access

switchport access vlan 10

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

end

Verify:

show port-security interface fa0/1

30. Port Security Verification Commands

Show port security

show port-security

Show specific interface

show port-security interface fa0/1

View learned MAC addresses

show mac address-table

View interface status

show interfaces status

Check error-disabled ports

show interfaces status err-disabled

31. VLAN + Access + Trunk + Port Security

These concepts work together.

Example:

VLAN 10

│

PC1

│

Fa0/1

│

┌───────┴───────┐

│     SW1       │

└───────┬───────┘

│

│ Fa0/24

│ TRUNK

│

┌───────┴───────┐

│     SW2       │

└───────┬───────┘

│

Fa0/1

│

PC2

VLAN 10

Configuration concept

PC1 → Access Port → VLAN 10

│

↓

Trunk

│

↓

VLAN 10

│

↓

Access Port

│

↓

PC2

Port security can be applied to the access ports where end devices connect.

32. Important CCNA Terminology

VLAN

A logical Layer 2 network/broadcast domain created within a switched network.

Access Port

A switch port assigned to one VLAN and normally used for an end device.

Trunk Port

A switch port that carries traffic for multiple VLANs.

Trunking

The process of carrying multiple VLANs over one physical link using VLAN tagging, commonly 802.1Q.

DTP

Cisco protocol used to dynamically negotiate trunking.

Port Security

A switch feature that restricts which source MAC addresses can use a port.

Sticky MAC

Allows the switch to dynamically learn MAC addresses and use them for port-security enforcement.

⭐ Final Concept Diagram

SWITCH

│

┌─────────────┴─────────────┐

│                           │

ACCESS PORT                 ACCESS PORT

│                           │

VLAN 10                     VLAN 20

│                           │

Student                    Trainer

│

Port Security

│

│

└───────────┐

│

TRUNK

│

VLAN 10 + VLAN 20

│

▼

SECOND SWITCH

The four concepts students should remember:

VLAN

↓

Separates the network logically

ACCESS PORT

↓

Connects an end device to one VLAN

TRUNK PORT

↓

Carries multiple VLANs between network devices

PORT SECURITY

↓

Controls allowed MAC addresses on a switch port