1. Virtual Trunking Protocol (VTP)

VTP = VLAN Trunking Protocol

VTP is a Cisco proprietary protocol used to manage VLANs across multiple switches.

🔹 Why VTP?

Without VTP:

SW1 → Create VLAN 10

SW2 → Create VLAN 10 manually

SW3 → Create VLAN 10 manually

With VTP:

SW1 (VTP Server)

↓

VLAN 10

↓

SW2 -------- SW3

(Client)     (Client)

👉 VLAN is automatically shared.

🔹 VTP Modes

1. Server Mode

- Can create, modify, delete VLANs

- Sends updates to other switches

vtp mode server

2. Client Mode

- Cannot create VLANs

- Receives VLAN information from server

vtp mode client

3. Transparent Mode

- Does NOT participate in VTP updates

- VLANs must be created manually

- Forwards VTP messages

vtp mode transparent

🔹 VTP Domain

All switches must have the same:

VTP DOMAIN NAME

vtp domain CCNA-LAB

🔹 VTP Password (Optional)

vtp password cisco

🔹 VTP Important Concept

Revision Number

- Every VLAN change increases revision number

- Higher revision = newer database

⚠️ If a switch with higher revision joins → it can overwrite VLANs

🔹 Best Practice

- Use Transparent mode in modern networks

- Avoid VTP in production unless controlled

2. Loop Avoidance

Loops happen when multiple paths exist in Layer 2 networks.

🔴 Problem Without Loop Control

SW1

/   \

SW2 — SW3

Loop causes:

❌ Broadcast Storm

Frame keeps looping infinitely

❌ MAC Table Instability

Switch cannot learn correct MAC addresses.

❌ Network Crash

High CPU + congestion

3. Spanning Tree Protocol (STP)

STP = Spanning Tree Protocol

STP prevents loops by blocking redundant paths.

🔹 STP Working

SW1 (Root)

/        \

SW2        SW3

\        /

BLOCKED

STP:

- Keeps one active path

- Blocks others

🔹 Key STP Concepts

Root Bridge

- Central reference switch

- All paths calculated from root

spanning-tree vlan 1 priority 4096

(lower priority = higher chance to become root)

Port Roles

| Role | Description |
| --- | --- |
| Root Port | Best path to root |
| Designated Port | Forwarding port |
| Blocking Port | Prevents loop |

🔹 Port States

Blocking → Listening → Learning → Forwarding

| State | Function |
| --- | --- |
| Blocking | No traffic |
| Listening | Prepares |
| Learning | Learns MAC |
| Forwarding | Sends traffic |

4. STP Modes

Different versions of STP exist.

🔹 1. STP (802.1D)

- Original STP

- Slow convergence (~30–50 sec)

🔹 2. RSTP (802.1w)

- Rapid Spanning Tree

- Faster convergence (~1–2 sec)

🔹 3. PVST+

- Cisco version

- Separate STP per VLAN

🔹 4. Rapid PVST+

- Combines RSTP + PVST

- Fast + VLAN-based

🔹 Recommendation

Use Rapid PVST+ in Cisco networks

5. PortFast

PortFast is used for access ports (PC connections).

🔹 Problem

Normal STP delay:

30 seconds delay before forwarding

🔹 Solution

spanning-tree portfast

Port goes immediately to:

FORWARDING STATE

🔹 Example

PC ─── Switch

(PortFast)

No waiting → faster connectivity

🔹 Important Rule

Use ONLY on access ports

❌ Do NOT use on trunk links

6. BPDU Guard

BPDU = Bridge Protocol Data Unit

Used by STP to exchange information.

🔹 Problem

If someone connects another switch:

Unauthorized switch → sends BPDU

Can affect STP topology

🔹 Solution

Enable BPDU Guard:

spanning-tree bpduguard enable

🔹 Behavior

BPDU received on PortFast port

↓

Port goes to ERR-DISABLED

🔹 Example

PC ─── Switch (PortFast + BPDU Guard)

│

❌ Unauthorized switch connected

↓

Port shutdown

7. PortFast + BPDU Guard Together

Best practice:

interface fa0/1

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

8. EtherChannel

EtherChannel = Link Aggregation

Combines multiple physical links into one logical link.

🔹 Without EtherChannel

SW1 ===== SW2

|         |

|         |

Multiple links (STP blocks some)

Some links are unused due to STP.

🔹 With EtherChannel

SW1 ======= SW2

Po1 (Port-Channel)

Multiple links act as one logical link.

🔹 Benefits

- Increased bandwidth

- Redundancy

- Load balancing

- STP treats as single link

🔹 EtherChannel Example

Fa0/1 + Fa0/2 + Fa0/3

↓

Port-Channel 1

9. EtherChannel Modes

Two main protocols:

🔹 1. PAgP (Cisco)

auto

desirable

🔹 2. LACP (IEEE Standard)

active

passive

🔹 Recommended

Use LACP (standard)

10. EtherChannel Configuration (LACP)

interface range fa0/1 - 2

channel-group 1 mode active

Then:

interface port-channel 1

switchport mode trunk

11. EtherChannel Requirements

All ports must have:

- Same speed

- Same duplex

- Same VLAN

- Same trunk/access mode

Otherwise:

❌ EtherChannel will not form

12. Verify EtherChannel

show etherchannel summary

13. Complete Concept Flow

VLAN

↓

Logical separation

TRUNK

↓

Carries VLANs

VTP

↓

Manages VLANs

STP

↓

Prevents loops

PORTFAST + BPDU GUARD

↓

Protect access ports

ETHERCHANNEL

↓

Increases bandwidth + redundancy

⭐ Final Understanding Diagram

VLAN 10 / VLAN 20

│

┌────┴────┐

│   SW1   │

└────┬────┘

│

EtherChannel

(Trunk Link)

│

┌────┴────┐

│   SW2   │

└────┬────┘

│

Access Ports

│

PC (PortFast + BPDU Guard)

STP ensures no loop

VTP manages VLANs

EtherChannel increases bandwidth

🎯 Key Exam Points

VTP

- Shares VLANs

- Modes: Server, Client, Transparent

STP

- Prevents loops

- Uses Root Bridge

PortFast

- Immediate forwarding

- Only for access ports

BPDU Guard

- Protects from rogue switches

EtherChannel

- Combines links

- Use LACP (active/passive)